## EXECUTIVE SUMMARY

The report describes the background to the choice of the dam site, the site conditions, topographical and geological features which have led to the selection of the type of dam and the detailed design thereof.

The initial design was subject to a design review by Hart Frost Consulting Engineers of Zimbabwe in 1998. As a result the yield of the dam was reassessed.

Because of the physical difficulties in investigating the foundations, and the complexities of these it was understood that the initial design would be reviewed on an interactive basis with exposure of the actual foundations during excavation. Exploratory excavations were started in October 2002 and completed in October 2003, to determine the hard rock horizon.  Although this process continued for most of the above period the only changes necessary were to amend the dam alignments and plan shape to ensure that it would be founded wholly on the banded ironstone rock formation which dominates the site.  This required a small reshaping of the arch in section.

Diamond drill holes were drilled on the extreme end of the abutments and on the crown of the arch. These exposed the fact that old mining adits which extended under the dam had previously been incorrectly surveyed. This necessitated a rehabilitation of the adits and an accurate survey of their positions. The survey showed that it would be necessary to plug the adits and ensure their water tightness upstream of the dam wall.

A further Design Review had been requested in November 2002 and Arup (Pty) Ltd. of South Africa was appointed.  Expert geological and geotechnical advice was sought in the final location of the dam and in the measures required to plug the adits. The review panel was kept informed of decisions.  Details are included in the geological and geotechnical reports which are appended to this design report

On the request of the Review Panel, analyses were completed to assess the impact of a temperature drop. Analyses incorporating temperature drop loading, using a finite element model set up by the designer, confirmed the acceptable behaviour of the final design arch configuration under these and all other loading conditions.

The dam as now designed comprises a masonry arch dam 40.0 metres high from lowest foundation level to the non overflow crest level.  This makes Lucilia Poort dam the highest rubble masonry arch dam in Zimbabwe and possibly the world  but the design stresses in the masonry have been kept at levels which have been successfully used on lower dams without any problems.

The active storage of 3.988Mm3 is provided by the top 25.5 metres of water in a reservoir with a full supply area of 33.75 hectares and a capacity of 4,126 Mm3.  The yield of the dam is estimated to be 1.83 Mm3 annum at a risk level of 4%.

Masonry, without construction joints, has been used extensively in Zimbabwe for arch dam construction but it is not a well known technique and therefore the design philosophy which led to the use of masonry for construction purposes and the shaping of the dam is described in detail.  The objective has been optimisation of the design to enable full use to be made of local resources in materials, manpower and equipment to construct an economical and safe structure.

The dam was designed using the Crown Cantilever Adjustment method to refine the shape.

A finite element model of the selected shape was then analysed to confirm that the stresses and deflection under extreme loading conditions were satisfactory and within safe limits.

The calculation methods and procedure are described in detail and the calculations and background information are attached as the appendices to the report.

Essential dam data is summarised below.

DATA

NAME OF DAM					LUCILIA POORT DAM

LOCATION						Map Reference SP 829 326

RIVER						Dwimbika

CATCHMENT AREA				92.8 Km2

MEAN ANNUAL RAINFALL			900 mm.

MEAN ANNUAL RUN OFF			10.208 Mm3/annum

RESERVOIR SURFACE AREA AT H.F.L.	40.00 hectares

RESERVOIR SURFACE AREA AT F.S.L.	33.75 Hectares

RESERVOIR CAPACITY AT F.S.L		4.126 Mm3

HEIGHT OF DAM					40.0 metres

SPILLWAY LENGTH				56 metres

DESIGN (2000 YR) INFLOW FLOOD 		675 m3/sec

DESIGN (2000 YR) OUTFLOW FLOOD		651 m3/sec

HIGH FLOOD LEVEL (H.F.L)			1237.95

TOTAL CREST LENGTH				156 metres

NON OVERFLOW CREST LENGTH                  100 metres

NON OVERFLOW CREST LEVEL                     R.L. 1238.00

LOWEST FOUNDATION LEVEL			 R.L. 1198.00

FULL SUPPLY LEVEL (F.S.L.)		            1235.0

LOWEST DRAWDOWN LEVEL (L.D.D.)            1209.5

WIDTH OF WALL AT BASE                                8.0 m.

WIDTH OF WALL AT CREST                              4.0 m.

YIELD AT 4% RISK				            1.830 Mm3/annum

1	INTRODUCTION AND BACKGROUND

After an extensive examination of the Umtebekwe and Umtebekwana Rivers and preliminary studies of alternative dam sites it was concluded that a dam on the Lucilia Poort dam site on the Dwimbika River, a tributary or the Umtebekwe River offered the cheapest means of satisfying both the long and short term water requirements for the Unki Mine development.

In 1997 preliminary topographical surveys and a geological survey of the site were undertaken and a geological report presented in May of that year.

In 1998 a design report was presented to Anglo American Zimbabwe proposing the construction of a masonry arch dam. The design requirement was that the dam could be built in two stages to allow for the phased development of the Unki Mine.

In June 1998 Anglo American Zimbabwe asked for an independent review of the design by a local Consulting Engineer, Hart Frost Consulting Engineers.

Arising from that review a more detailed assessment of the dam yield was made and constructive amendments to the finite element analysis of the dam, which was then in progress, were recommended and implemented.

Attention was drawn to the need for a more detailed geotechnical survey to be undertaken. This matter had already been discussed with Anglo American Zimbabwe but due to land ownership problems, up to that point, access for further land clearing, survey and excavation had been denied. Hart Frost Consulting Engineers concurred with the SSZ view that as soon as access was granted the site should be cleared, the dam accurately set out and the soft overburden be stripped to expose the general hard rock profile. Any adjustments to the alignment of the dam could then be made and then a limited number of diamond holes should be drilled to prove the foundations at depth.

Between September 2002 when access was granted to the site and July 2003 the foundations for the dam were exposed and the arch alignment both moved and reshaped to suit.  Details of this are described in Appendix 1.  Initial calculations were carried out using the Crown Cantilever method and once the final excavated profile was established the dam stresses were analysed using Algor.  This confirmed to a large degree the results of the Crown Cantilever analyses.

The location of the adits below the dam were also resurveyed and it became apparent that parts extended into the basin and that they needed to be sealed.  The left abutment in particular is jointed and during the construction decisions were made to stabilise this under and upstream of the arch with grouted anchor bars.  Details of the above are included in Section 9 and also the Construction Report

Drawing No. Z09744WB/13 shows the foundation excavations for the final alignment which was accepted before any masonry was placed. The line of the initial spiral of reference is shown, to indicate the changes in position to achieve the optimum alignment.

The increase in the crown radius required a slight thickening of the arch from 5.5 to 6.0 metres over the constant thickness section and a small modification of the downstream toe section to maintain the same stress levels as for the original design.

In November 2002 Anglo American Zimbabwe had called for a further review of the Design by an International firm of Consulting Engineers. Arup (Pty) Ltd of South Africa was appointed and they in turn appointed other consultants to review different aspects, Chris Brooker and Associates (cba) were appointed to review the Hydrology, ARQ to review the structural aspects of the dam and Schwartz Tromp and Associates to review the geotechnical aspects of the dam.

Comments were made and have been addressed where required in the body of this report.

The design of the dam was finalised with only some foundation details at the upper abutments to be confirmed in the light of the final foundation excavations.

## 2	LOCALITY AND ACCESS

The dam site is located about 300 metres below the confluence of the Dwimbika River and the Home Farm Spruit as shown on the Locality Plan (Drawing No Unki/W1/001).

It lies roughly 11 kilometres north of Shurugwi. One access road leads from Guinea Fowl on the watershed to Valley Mine. There is a turn off to the left just beyond Guinea Fowl School on the Gweru to Shurugwi main road. The road follows the watershed most of the way. The road turns right beyond the Valley Mine and follows the settlements on the upper west slope of the Umtebekwe River valley. After crossing the Dwimbika River the road turns North for about a kilometre to the turn off to the Dam site on the right. The newly  constructed dam access road crosses a right bank tributary of the Dwimbika River and follows along the foot of a range of hills a distance of about 1½ km to the Contractor’s construction camp. From there a track, 600 metres long, runs along the right bank of the Dwimbika River, which leads directly to the dam site.

A second access road leads off the Shurugwi Zvishavane road. Proceeding from Shurugwi to the south for about 7 km. A road leads to the left about 1 km beyond the foot of the Wolfshall Pass. This is the main road to Unki Mine. After about 7¾ km proceed northwards beyond the Unki Mine turn off and at approximately 16½  km from the main road turn left at a T junction and almost immediately turn right onto the dam access road and thence to the Contractor’s camp and the dam site.

There is a third access road with precipitous side slopes and subject to land slips in wet weather. It is regarded as dangerous but is a very short route to Shurugwi from the site. Coming from the dam, turn right at the end of the access road and follow the road through Magakooshla Annexe which leads directly to Shurugwi after about 8 km.

# 3	CATCHMENT AND RAINFALL

The Catchment Area has been measured on the 1 in 50 000 maps and is estimated to be  92.8 Km2.  The length of the longest watercourses have been measured, namely the Home Farm Spruit with headwaters at Guinea Fowl at about elevation 1465 mAMSL with a length of 14.5 km and the Dwimbika River with its headwaters in the Wildebeeste Block at about elevation 1460 mAMSL and a length of 11.2 km.

The rainfall records show a mean annual rainfall of 1023 mm for Selukwe (now Shurugwi) about 10 km south west of the site. The rainfall maps show an area of high rainfall with its eastern edge roughly along the Guinea Fowl - Shurugwi ridge of high ground and extending eastward to the Umtebekwe River. The catchment area of the dam lies within the isohyets 800 mm to 1000 mm at Shurugwi.

A mean annual rainfall of 900 mm for the catchment has been accepted.

This region of high rainfall is well established from records going back to 1916. It is attributed to the very steep hilly country between the Umtebekwe River and the Shurugwi – Guinea Fowl watershed area, where the ground rises from elevation 1200 mAMSL at the dam site to elevation 1 440 mAMSL over a distance of about 5 km.

Figs 5 and 6 show isohyets from two maps, one as published in 1946 in inches and a later map published in 1984 in millimetres.

### 4	THE RESERVOIR

The dam site is located on the Dwimbika River about 300 metres below the confluence of the Home Farm Spruit. The basin extends up both streams in steep sided broken valleys. The land is tree covered up the banks and is in a natural untouched state.

The basin lies entirely within the metabasalts of the Tibilikwe geological formation. Several ironstone ridges, running roughly North South, cut across the basin.

In the Geological Report by T.J. Broderick of May 1997 he notes that “ the Home Farm Spruit and the Dwimbika River have created gorge like breaks through the resistant ironstone ridges”. The 1 in 1 000 and the 1 in 2 500 scale aerial mapping of the site and the basin show the rugged nature of the terrain and this is even more vividly portrayed by the air photographs. In general the area is rocky and the soil cover is shallow.

There is little likelihood of the underlying rock masses becoming unstable due to their inundation by the lake waters. Because the area is a high rainfall area they will already have been saturated at some time and reached a stable slope.  The worst that is likely to happen is that shallow surface slips in the soil layer may occur in areas of steep slopes. It should be noted that the dam cannot be subject to rapid drawdown, which is the main trigger for such events, under normal operation,. Shallow slips are not likely to give rise to large waves, the volume of material being small in relation to the reservoir capacity.

Wave action due to wind will be minimal because of the winding shape of the rather narrow arms of the rivers which limit the extent of fetches of open water.

The surface areas have been  planimetered from the air survey mapping of the basin with 2 metre contour intervals and the areas at the full supply level of R.L. 1235.00 and high level flood level of R.L. 1238.00  are 33.75 hectares and 40.00 hectares respectively.

The corresponding capacities are 4,126 Mm3 and 5,210 Mm3 respectively.

The capacity curve can be expressed as an equation in the form

Capacity  =  A(Z – Zb)B

Where A and B are constants and Z is a given water level and Zb is a base level.

The constants are derived by a linear regression of the Log values of depth and capacity calculated from the surface areas.

In this case the equation becomes

Capacity  =  190.18578(Z – 1198.82)2.78247

The purpose of expressing the capacity in this way is to make the depth capacity relationship easy to manipulate mathematically in yield, flood routing and discharge / depth calculations and in predicting the behaviour of the dam.

The capacity curve calculation, a depth / capacity table and surface area and capacity curves are attached as figs 1 to 4.

It is not proposed to bush clear the basin and the reason for not doing so is quoted below from the advice of Dr Brian Marshall, an experienced environmentalist.

“The trees decay slowly so they do not effect water quality. They also provide “structure” to the underwater environment, increase the substrata available for algae growth and invertebrates and provide cover for small fishes. This enhances fish productivity and the general view now is that trees are beneficial for fishes, anyone who goes fishing at Kariba will know that most fish are caught around trees. Another advantage of trees is that they damage nets and prevent fishing from taking place in some areas – thus controlling fishing effort without aggravation!”

The outlet level of the pipes delivering water to Unki Mine will be set at about level

R.L. 1209.50.

The passage of normal flow will be from a low level 200 mm diameter pipe set at about R.L.1200.60. This will discharge de-oxygenated water from the dam into the river which will quickly become aerated by passage over the low waterfall below the dam and over the gauging weir some distance below the dam.

## 5	FLOOD CALCULATIONS

In Zimbabwe the Mitchell Formula has been used almost universally since 1973 when T.B. Mitchell of the Hydrological Branch of the Ministry of Water Development published the results of his research as “A Study of Rhodesian Floods and Proposed Flood Formula”, later reproduced in The Rhodesian Engineer Paper No 160 in 1974, a copy of which is attached. (Appendix 2).

Mr Mitchell reviewed his paper in another paper entitled “Annual River Frequency in Zimbabwe” published in the Zimbabwe Engineer October 1998, copy attached. (Appendix 3).

Some 200 Large Dams and several thousand smaller dams in Zimbabwe have been designed and constructed in accordance with the requirements of this formula which has proved to be very reliable.

It is the accepted formula for the Dam Safety Section of the Zimbabwe National Water Authority and is used by all dam engineers in Zimbabwe.

The design criteria used in Zimbabwe for Flood and Hazard evaluation are summarised in a Paper (No 249) entitled “Assessment of Design Flood Hydrographs” by T.C. Kabell.  (Appendix  4).  The dam is a major dam with a moderate hazard potential and is therefore a Class 1 dam to be designed to pass the 2 000 year flood with dry freeboard and the 10 000 year flood without freeboard.

On the basis of the foregoing the estimated 1 : 2000 year return period flood of 675 m3 sec has been used as the design flood.  The inflow flood hydrograph has been estimated, based on the catchment parameters,  using the procedures outlined in the above paper.  This has then been routed through the reservoir assuming that the dam was full at the start of the flood event.

As a comparison and at the suggestion of the review panel other flood formula have been considered. Fig. 7 Summarises the Maximum Probable Flood Calculation using 3 different formulae.

According to our copy of TR 137 – Regional Maximum Flood Peaks in Southern Africa by Z. Kovac’s (Dec 1988) the catchment for Lucilia Poort falls well within Maximum Flood Peak Region 5 (see Fig 13 attached).The reviewers have used figures of 5.2 and 5.6 which is questionable. It makes a difference of 72.% to the value of the SED. It should also be noted that the boundaries of the regions in Zimbabwe are shown dotted and they are therefore  tentative.

It does not seem reasonable to increase the Mitchell 1 in 10 000 year flood of 911 cumec by a factor of 2.44 when the Mitchell Formula is apparently satisfactory.

Even in the event of such a prodigious flood as 2198 cumec taking place it would not present a threat to the stability of the arch dam. A rough check indicates that stresses would not be increased significantly if the non-overflow sections of the arch were overtopped leading to a total flood rise of 5.3 metres above the dam full supply level. (see the Crown Cantilever adjustment calculations in Appendix 11 -14.)

Discharge over the non-overflow section would be on to solid rock or masonry and the extreme flow duration would not exceed 3.6 hrs. See flood routing calculations and graphs. attached. Figs  8, 9, 10 and 11.

## 6	RUN OFF AND YIELD

The run-off records from the nearest gauging stations are used for the yield studies but there is not sufficient data to calculate a realistic rainfall/run-off relationship.

Following the observations of the Hart Frost review of 1998 the yield calculations were again reviewed accepting the run off records of the Umtebabwe Rietfontein E-42 gauging weir but modifying the unit mean annual run off (UMAR) and coefficient of variation (CV) figures after discussion with the hydrologists of the hydrological Branch of the Department of Water Development who were familiar with the area.

Fig 12 shows the relationship between the rainfall at Shurugwi and the run off at the Umtebekwe gauging weir.

The Umtebekwe River catchment lies in an area of lower mean annual rainfall than the dam site catchment and has rather different topographical and geological characteristics. It was considered that the unit mean annual run off of 110 mm and CV of 1.1 accepted, was probably conservative.

The Hydrological Branch of the Ministry of Water Development has done a very substantial amount of research in the past and the engineers engaged in it are of international standing, in particular T.C. Kabell and T.B. Mitchell.

They have concluded that the Weibull probability distribution is the most appropriate for Zimbabwe run off characteristics and it is agreed that yield studies should be based on that distribution.

The yield of a dam is the constant amount that can be supplied from it at a given risk of failure to supply (ie. For the x% yield, the stated yield would not be met, on average, in x years out of 100).

Failure of supply of the stated yield does not necessarily mean nil supply of water, but anything less than the stated yield.

The calculation of the yield is carried out by generating a run-off sequence over a long period which will supposedly have the same UMAR and CV as the recorded data. The behaviour of the reservoir is then observed under the generated sequence of run off and assumed monthly values of rainfall, evaporation, demand and release of normal flow.

For a given full supply level (FSL) and capacity, and a known relationship between reservoir level and capacity, the percentage of failure years is observed. The FSL is then adjusted and the trials re-run until the required risk level is achieved.

Excel programmes have been written to calculate the run off probabilities for catchments with a given M.A.R. and C.V. and to undertake behaviour trials for dams. A calculation for the run off probabilities using an MAR of 110 mm. and a CV of 1.1 is given in Fig.14.These are the values accepted following the Hart Frost review.

The difference to the probability distribution curve as a result of varying the value of C.V. is shown on Fig 15.

A re-evaluation of the yield was carried out in February 1999 allowing for the passage of 50 litres/second or normal flow, whichever the less.

The 4 % yield was estimated to be 1.83 million cu.m./annum and the risk for a yield of 2.5 million cu.m /annum was assessed as 11.5 percent.

It is considered that the results obtained from the calculations are realistic.

The results of the yield programme generally agree well with results obtained from T.B.Mitchell’s transition probability matrix method.

The behaviour trial method is preferred because of its ability to clearly illustrate the behaviour of the system.

The detailed calculations for the yield are attached as Appendix 5.

### 7	GROUTING AND SILTATION

  - Grouting

The dam foundations comprise solid ironstone except for the upper abutments where the rock is fractured to some depth.  The ironstone is generally massive and water tight but tight joints and shear planes do exist.  Fissures are generally groutable and a single grout curtain with holes at 6 m spacings to a depth of 12 metres will be provided.  The object of the grout curtain is to seal leakage through the foundations.  The grouting will be monitored during construction and where grout takes are high and in some specific areas e.g. close to the adits additional grouting will be carried out.

It is recognised that the upper abutments are the most vulnerable to seepage and extensive grouting and anchoring is being undertaken to seal the more jointed rock in these areas.

7.2	Siltation

Attempts have been made to standardise the assessment of sediment yields according to certain criteria, which are described in a paper entitled Sediment and Yield of Small Dams by Eng TC Kabell.(Appendix  6)

In the end it becomes a matter of personal judgement considerably aided by the above paper.

In the case of Lucila Poor Dam, where the catchment is regarded as well conserved and in a natural state it is concluded that it is not subjected to excessive erosion.

The sediment yield from the catchment has been assessed as 300 tonnes/km2/annum.

The storage ratio, defined as the storage capacity divided by the mean annual run off is 0.40.  The reservoir trap efficiency is therefore regarded as 100% ie all the silt entering the reservoir will remain there.

Most of the silt becomes deposited in the upper reaches of the dam and the behaviour studies indicate that the average water level will be at 1233 mAMSL, which confirms that that will be the case.

The calculation of the total silt deposition is given below for a 20 year period.

| Catchment Area | 92.8km2 |
| --- | --- |
| Unit M.A.R. | 110 mm |
| Mean Annual Inflow | 10 230 Mm3 |
| Dam Capacity | 4 126 330 m3 |
| Storage Ratio | 0.403 |
| Trap Efficiency | 100% |
| Sediment Yield | 300 Tonnes/Km2/annum |
| Average Annual Silt Deposit | 27 840 tonnes |
| Density of Silt | 1.55 Tonnes/m3 |
| Average Annual Silt Deposit | 17 961m3/annum |
| Total Deposition in 20 years | 359 226m3 |

If it is assumed that 20% of the silt yield reaches the dam wall it would take up to 38 years for the silt to reach the level of the outlet pipes.

Perhaps 30% of the silt volume would be extractable water so that the actual loss not available for yield purposes would be about 252 000 m3.

Behaviour trials indicate that this might increase the long  term yield risk level by 3% after 20 years. Any attempt at a precise calculation would be highly academic, based on data which are far from precise.

It is concluded that siltation is not a serious problem provided that the character of the catchment does not change.

### 8	TOPOGRAPHY

The dam site is located at a point where the Dwimbika River has cut through a 40m wide massive banded ironstone dyke resulting in a flat river bed of solid rock. The down stream edge of the dyke is marked by an approximately 2 metre high, sharp fall in the river bed.

The left bank comprises a rocky spur with outcrops of solid rock in places. The side slopes are steep at about 1.75 horizontal to 1 vertical becoming very steep at about 1.25 to 1 near the proposed dam crest.

The right bank is less steep at the lower levels with a slope of about 1.75 to 1, at about the dam mid height the slopes become rocky and precipitous.

The lower slopes are generally covered by soil and scree from the hillside to a depth of about 3 metres.

A sharp jagged ridge of solid rock juts out on the right bank downstream of the proposed dam centreline.

The developed section on the proposed dam centreline is seen as a flattened asymmetric U shape.

### 9	GEOLOGY OF THE DAM SITE

9.1	General

The geology of the site was first described by T.J. Broderick of Jeremy Prince & Associates (Pvt) Ltd in his Report dated  6th May 1997, Ref. 1 in the list of reports attached.

The dam site is an approximately 40m wide massive banded ironstone formation with a roughly north south strike which dips to the upstream (west) at an angle of about 75 degrees.

The ironstone formation is overlain by jointed and blocky greenstone to the upstream.  In the river bed the downstream edge of the ironstone is marked by the low water fall.  The downstream footwall formations comprise phyllonites which are massive.

It was originally envisaged that the arch would be founded on the ironstone but may be extended on the upstream crown across the greenstone contact area in order to reduce the arch radius.

In the event the greenstone proved to be highly weathered on the contact with the ironstone and it was necessary to amend the alignment to ensure that the dam footprint is entirely on competent banded ironstone.

On the upper left bank a shear zone underlain by soft phyllonite rock was exposed and it was necessary to realign the arch by a few metres to the upstream to bring the abutment on to the ironstone formation.

The rock at the left bank crest is rather fissured but at this point the arch thrusts are low and it is considered that the rock mass can be rendered monolithic by grouting and anchor bars. The final decision was left until the height of the wall allowed relatively easy access to the area and final clearing was complete. 

As a result of the mapping of joints and an analysis of the stability of the abutments it was considered that there may be a potential for surface blocks on the left abutment upstream of the wall to slip.  As a precaution anchor bars were grouted into this slope.

A report summarising the geology of the site as revealed during the construction with sections was produced in May 2005 by T J Broderick and is included with this design report because it assists understanding the geology of the site and in appreciating the decisions taken in moving the arch and in the grouting of anchors on the left abutment.  (Ref 9 in the list of geological reports).

In this initial report Jeremy Prince & Associates note that the central area of Zimbabwe is generally regarded as aseismic, only few small events exceeding a magnitude of 2.8 on the Richter Scale having been recorded. This is confirmed by a paper “Earthquakes in Zimbabwe: an engineering concern”, by E. Jonathon published in Proceedings of the Zimbabwe Institution of Engineers in January 1999 attached as Appendix 7

9.2	The Adits

It was known from the start that old mining adits had been driven on both banks of the river at about level 1206 mAMSL, that is about 6m above the river bed at the site. The left bank adit portal is located about 45 m downstream from the waterfall and the right bank portal is located about 35 m down stream from the waterfall.

Initial surveys had shown drives running approximately north and south but with branches running roughly parallel to the river upstream. These surveys showed that these branches ended at about the line of the dam.wall.

An exploratory diamond drill hole on the extreme end of the arch on the left bank penetrated an adit but not in a place where it was expected. The adits which had fallen into a state of disrepair were then rehabilitated and an accurate survey was made which showed that the upstream trending North bank adit extended some 33 metres upstream of the dam wall, terminating in an area of sheared and decomposed rock in the greenstones. From the air survey mapping it appears there was only about 15 metres of cover above the adit roof.

It was clear that this would present a serious problem of leakage into the adit with the strong probability of piping developing from the basin.

It was decided to completely fill the adit with masonry from the end up to the upstream line of the dam wall. Following the comment of the Geotechnical Review Report (ref.8) the filling and plugging of the adit was extended a further 5 metres to the downstream side of the dam wall.

It is considered that the sides and bottom of the adit can be made watertight with a masonry plug but there is a danger of some settlement of mortar along the roof of the adit and arrangements have been made to place a continuous slotted pipe along the contact area, with rubber covers over the slots to allow circulating grout to feed through the slots but not backwards. The circulating grout system will enable the grout pipe to be washed out and the operation to be repeated if necessary.

There is a similar upstream branch adit on the right (south), bank adit which extends about 45 m beyond the extended line of the right bank arch but it is about 50 m into the bank beyond dam full supply level and has about 23m of ground cover above the roof. The adit is in good solid rock up to the last 4m or so where there are some shear fissures evident. It has been decided to plug the last 13m of this adit with masonry in the same way as for the north bank adit.

It is planned to grout the sides, top and bottom of the north bank adit plug by a system of vertical and inclined grout holes drilled from the rock foundation of the arch and intersecting the plug.

Leakage from the plug is expected to be minimal but if necessary further grouting could be carried out by drilling grout holes from within the adit, through the end face of the plug.

It is not expected that any grouting of the south bank end plug will be necessary but, if it is, it will be carried out through the plug face.

## 9.3	Geological Reports

The following geological Reports describe the evolution of the geological assessment of the site and give the background to the decisions taken. These reports are appended to this.  Additional geological and geotechnical reports are appended to the construction report.

1	Report on a visit to examine the Geology of the Lucilia Poort Dam Site

Dwimbika river, Shurugwi District by T.J. Broderick of Jeremy Prince and Associates – 6th May 1997

- Lucilia Poort Dam : Design Review – Preliminary Comments on Site Investigations by Hart Frost Consulting Engineers  -  5 October 1998

- Unki Mine – Lucilia Poort Dam – Possible Phase 1 Construction by Stewart Scott Zimbabwe.  27 June 2002.

3	Report on a visit to Lucilia Poort Dam Site  - 23  to 24 February 2003 by Jeremy Prince and Associates. 6 March 2003

- Report No 02/105/1  :  Lucilia Poort Dam, Shurugwi, Zimbabwe. Geotechnical Review / Site Inspection by Schwartz Tromp and Associates  -  9th June 2003.

- Comments on the Geotechnical Review / Site Inspection carried out by Schwartz Tromp  by Stewart Scott Zimbabwe.

- Report on a visit to inspect the final foundation excavations at Lucilia Poort Dam

23 – 24 July 2003 by Jeremy Prince and Associates  -  14 August 2003

- Report on a visit to assess the geology of the adits at Lucilia Poort Dam 5th – 6th September 2003  by Jeremy Prince and Associates–15 th September 2003

- Progress Report on the Geotechnical review by Stewart Scott Zimbabwe 22 September 2003

9	A Review of the Geological Setting at Lucilia Poort Dam, Dwimbika River,

Shurugwe following a site visit on 3rd and 4th May 2005

### 10	THE DESIGN PHILOSOPHY

10.1 	General

Lucilia Poort dam site has features which imposes some constraints on the design of the dam.

It is located in a relatively narrow steep sided rocky gorge with a rather flat, wide bottomed U shaped valley.

Given the adequacy of the abutments it is an obvious site for an arch dam.

It is necessary to found the dam on a banded ironstone ridge of limited width (approximately 40 metres.).

The depth to a good solid rock foundation was only determined after extensive excavations, which confirmed that the site did not lend itself to a double curvature shape with decreasing radii at the lower levels or to arches of variable thickness.

The exposed rock confirmed that the logarithmic spiral shape of the arches originally envisaged could best fit the site and give good angles of incidence throughout with the line of thrust generally being almost normal to the abutment rock contours.

The chosen alignment is symetrical about the spiral axis but a little asymetric to the valley axis.

The limitations imposed by the site constraints confirmed that a suitable vertical section for the crown cantilever with constant thickness arches would lend itself to masonry construction.

Although the crest length to height ratio is not excessive at 154/40 = 3.85, experience has shown that tension stresses will develop at the upstream base of the crown cantilever. This may result in limited cracking, probably at the rock / masonry interface, which will lead to tensions in the downstream face at about mid height.

The advantage of an arch dam is that it can make the best use of concrete or masonry by fully utilising the compressive strength of the material by transferring the imposed water loads to the abutments.

Design is directed to the shaping of the structure to achieve uniformity of compressive stresses and the minimisation of tensile stresses with the objective of minimising the quantities of material.

These objectives can generally only be achieved by complex shaping aimed at reducing the radii of arches and shaping the vertical section to allow gravity forces due to self weight to counteract vertical tension forces.

However, minimising quantities does not often result in the minimisation of costs as it leads to complications of construction, the costs of which, commonly exceeds the savings in the cost of materials.

The best economy can usually be achieved by designing the structure for simplicity and ease of construction and flexible programming of the works.

The greatest economy is often achieved by saving time, rather than material and making maximum use of readily available resources of materials, skills, manpower and equipment.

The design strategies to minimise quantities and achieve an even stress distribution are always in mind but must be integrated with all the other considerations.

It is these considerations which have led to the extensive use of masonry for arch dams in the Zimbabwe context.

10.2	The Case For Masonry Arch Dams

Masonry arch dams have been constructed country wide in Zimbabwe over the last 20 years. They were inspired by the skills of local contractors who had constructed many masonry gravity dams over the previous years and by the economies that resulted from this type of construction. Many of the factors listed below, which favour the construction of masonry arch dams, may be peculiar to Zimbabwe and may not be applicable in other countries.

- Zimbabwe generally has an equable climate without extremes of cold weather, This together with the methods of construction have shown that arch dams can be constructed without any radial contraction joints which are a common feature of concrete dams. Of the many dams that have been built none have reported the development of radial cracks.

- The construction expertise has been long established and it has been shown that local masons using locally quarried rock can produce good masonry with an acceptable and aesthetically pleasing finish.

- One of the predominant rocks in Zimbabwe is granite, which, if correctly quarried, produces an excellent masonry rock of manageable size and good shape. Other rock can be suitable if un-weathered, hard, clean and with some reasonably plane surfaces.

- The fact that radial joints are eliminated and face shuttering is not required results in large cost savings.

- Although it is accepted that permissible compressive stresses, usually limited to 4 Mpa, in masonry are lower than those for concrete which are generally in excess of 6 Mpa and therefore, the width and volume of masonry must be greater than would be required for concrete, the cost of masonry is substantially less than that of concrete. There is a net reduction in cost by using masonry. Preliminary studies indicated that a masonry arch dam would be at least 30% cheaper than a concrete Arch.

- It is common with arch dams to arrange for a crest overflow spillway. A masonry arch tends to have a wide crest. This  is advantageous for the spillway design as it allows a flip bucket to be incorporated on the crest, which throws water clear of the wall, with greater energy dissipation.

- Masonry can be overtopped by floods during construction without damage whereas the shuttering for concrete is vulnerable to flood damage.

- Masonry can be placed at a more or less constant level at any stage, which makes for easy access. Concrete, on the other hand, is placed in blocks and it is easier for moving shutters if the blocks are allowed to have quite high differential levels but it complicates access along the crest.

- Masonry, being built without radial joints, requires no joint grouting representing savings in equipment and time and eliminates the construction programming restraints due to joint grouting.

- Batching and transporting mortar for masonry is easier and less severe on equipment than batching concrete.

- Masonry can be stopped and started at any time and there is no constraint to complete a pour, once having started it, as is the case with concrete,

- Programming is very much more flexible with masonry than it is with concrete.

- The use of masonry maximises the use of local expertise, local labour and materials and minimises the project cost.

In designing Lucilia Poort Dam although the height of the dam exceeds previously constructed dams the stresses have been kept within the limits which have been used on lower dams.

!0.3	Construction Materials

The construction materials comprise rock, sand, cement and water.  The quality of the materials required is described in the Contract Specifications.

Field investigations showed that rock and sand of suitable quality was not available close to the dam site but an outcrop of sound granite rock was located about 12 km, North of the site and large deposits of clean well graded granite derived sand were located on the Umtebekwana river some 20 km, south East of the site. Both sources  are located close to existing access roads.

Experience has shown that granite is the best material for masonry rock. It is hard and durable and when well quarried it  produces a well shaped cubic rock of a size which can be handled by the masons. Granite derived sands also comprise well shaped grains and ideally graded material, generally a little coarse but which can be blended with a finer pit sand of similar origin to produce an excellent mortar sand with a low water demand. The quality of both rock and sand virtually guaranteed that good quality, strong, impervious masonry could be produced.  Granite aggregates are noted in the applicable Zimbabwe Standards as been very suitable for concrete (masonry).

The water in the Dwimbika River at the dam site is normally clean, slightly hard water of good quality for making mortar.

Cement will be sourced from UNICEM in Bulawayo which had been found to be of proven good quality in other works.

10.4	The Prescribed Mix of Mortar for Masonry.

The prescribed mix is based on the assumption that the mortar will be a sand / cement mix of 4 / 1 by weight.

The compressive strength of the mortar is fundamentally a function of the water/cement ratio. The mortar depends on the water demand of the sand and the water, which must be added to achieve the workability required to make good masonry. In practice the amount of water to be added changes with variations in the moisture content of the sand and grading of the sand. This is best controlled at the mixer by visual observation of the consistency of the mortar.

In general the practice in Zimbabwe is to select a clean river sand (preferably of granitic origin) and to mix it with a finer pit sand since the river sand is usually too coarse on its own. The resulting combination of the sands usually has a fineness modulus of between 2.6 and 3.0.

With the dosage of cement as specified in the prescribed mix it is expected that the set mortar will have a compressive strength of at least 15 Mpa.  This conclusion is supported by a paper entitled “Aggregates and the quality and economics of concrete” by “Dr D.E.Davis” dated Dec.1981, Reference Figure 3. in that paper, which is attached as Appendix 8 to this report.

The prescribed mix has been used for the construction of many dams in Zimbabwe , comprising gravity dams, buttress dams, arch dams and appurtenant  works on earth dams including retaining walls, abutment blocks and, spillway structures.

The behaviour of the masonry on nearly 100 projects under the control of SSZ, has been very good and we are unaware of any problems with the many structures built by others.

It should be noted that the maximum design compressive stresses seldom exceed 4Mpa. and the factors of safety against failure of the masonry in compression are probably very high.

The tests on the materials and on the strength of the mortar cubes carried out at Lucilia Poort Dam. confirm the initial assessment of the quality of the materials and the mortar strength predicted

10.5	The Concept Of Arch Dams In Wide Valleys

The acceptance of the fact that arch dams may be built in relatively wide valleys  which in turn leads to the acceptance of joints or cracks in the vertical section of the dam, to relieve tension stresses which may develop, has led to the design of several internationally notable large arch dams in Zimbabwe and elsewhere, including numerous smaller masonry dams, particularly in Zimbabwe.

Andre Coyne may be considered to be the originator of the concept. Firstly by the design and construction of Moulin Ribou dam in France. It is a 16 metre high cylindrical arch dam with a constant thickness of 1.5 metres and a crest length of 162 metres. It is built in a U shaped valley and incorporates horizontal hinged joints at the base and at approximately 1/3 the height.(see Ref 3 in the Bibliography of papers on arch dams in wide valleys, Attached as Fig 77)

In 1955 Coyne et Bellier had designed and constructed Grangent dam, another notable constant thickness arch dam. It is 54 metres high with a crest length of about 195 metres and is 8.5 metres thick, see Ref. 3.

The successful performance of these dams led to the design and construction of the North bank cofferdam at Kariba. This was a cylindrical dam essentially of constant thickness of 1.83m and 19 metres high with a crest length of 271 metres The design concept and behaviour is fully described in Ref 10.

Kariba dam although of double curvature is cited as a leading example of the concept of thickening of the upper arches, as opposed to the concept of thickening of the lower arches as favoured by other designers in different circumstances. The two approaches are discussed by the General Reporters at the ICOLD congress in Rome in 1991 for Question No.26, “Modern techniques of concrete dams in wide valleys and ancillary works.” Ref.6,

Kariba dam is briefly described in Paper R 10 presented at the Congress as having a crest width of 13 metres and a maximum width of 20 metres at about 1/3 height and a base width of 18.5 metres on the crown of the arch. The lower arches are thickened at the springing to a maximum width of 26 metres. The crest length is 615 metres.  Kariba dam is the subject of numerous technical papers and the thick crest approach is often noted in these papers.

Bangala dam in Zimbabwe is another notable example, designed by Coyne et Bellier and constructed by the Ministry of Water Development in 1962. The dam site is in a wide U shaped valley. Seven alternative designs were studied and the Contractor, who had just completed the Kyle dam construction, was involved in the design process to obtain the most economical solution, which was not the least concrete solution.

The design chosen was a constant thickness cylindrical arch with a central overflow ski jump spillway, made possible by the thick crest of the dam. The height of the dam is 50.7 metres and the crest length is 396 metres. The thickness of the arch is 6.71 metres.

As is the case in all such dams of this type the crown cantilever adjustment showed very high tensile stresses at the upstream heel of the dam and appreciable tensile stresses on the downstream face. It was recognized that cracking or yielding of the concrete or the rock foundation must occur under full loading. But further analysis showed that cracking would be limited, as the water load was transferred to the upper arches. The effect of cracking of the downstream face was also studied and it became clear that the even in a cracked state the arches could safely carry the loads. Although the construction of a base joint was debated it was decided not to include one.

In the event the dam was completed very quickly due to the simplicity of the design and the concrete programming flexibility which the constant thickness shape allowed. The dam was subjected to a significant flood while work on the non overflow section was incomplete.

It has now been in service for forty two years and has experienced an exceptionally heavy flood in 1999. It has behaved well and cracking of the concrete is not evident.

At the same time as Bangala dam was being built a dam of similar dimensions was being built in Switzerland. The Schiffenen dam, built in a U shaped valley described in Refs18 & 27 is 47 metres high with a crest length of 417 metres the thickness varies from 7 metres at the crest to 9.5 metres at the base.

The behaviour of Bangala dam has led to confidence in the concept of thickening the top arches of a dam where appropriate and in the acceptance of high tensile stresses at the upstream base of the cantilevers which has been shown to give rise to cracking in numerous cases but which does not threaten the safety of the dam if it is otherwise appropriately proportioned.

11	ARCH ANALYSIS AND DESIGN

11.1	Preliminary Design

The paper , “The Design of Small Arch Dams in Wide U Shaped Valleys “ is attached as Appendix 9. This paper presents a limiting states approach based purely on considerations of statics. It assumes that the cantilevers are rigid and of constant thickness and crack at points of maximum bending moment and become hinged at those points.

Three states are considered

- The cantilever is cracked at the base (effectively the dam / foundation contact)

- The cantilever is cracked at the base and at the point of highest tension on the downstream face.

- Tan phi, the friction factor at the base exceeds 0,75 and sliding of the base occurs.

In each state movement stops when the arches take up the water loads transferred from the cantilever. The  bending moments become zero at the crack points and tan Phi becomes 0.75 again. The arch stresses based on the arch loads and the simple cylinder formula are then at their maximum possible value.

An Excel spreadsheet “CONARC” calculates the maximum arch loads given the Arch Radius, the depth to full supply level, the surcharged depth above full supply level and the arch thickness.

An example of the calculation and the curves showing the arch loads for the three states is given in figs. 17 and 18.

The radius, depth and surcharge are usually known and by varying the thickness until the maximum arch stress becomes about 300 tonnes/square metre, an initial estimate of arch thickness is made.

11.2	Crown Cantilever Adjustment Program

11.2.1	The Concept

An arch cannot carry a load without deflecting. At the base of an arch dam only the small deflections due to foundation yielding can take place due to the fixity of the dam into the foundation. Arch loading at the base is therefore small and the arch loading at the other levels depends on the stiffness of the dam section in a vertical plane considered as a cantilever.

In the “Trial Load Method “ developed by the U. S. Bureau of Reclamation, an arch dam is considered as an assembly of horizontal arch and vertical cantilever elements, each of which takes a portion of the load.

At the points of intersection the displacements of the cantilever due to the cantilever loads must equal the displacements of the arches due to the arch loads and the sum of the arch loads and the cantilever loads must equal the total external load.

For a complete adjustment , linear displacements in the radial, tangential and vertical planes , together with angular displacements in each plane must be equated for each point of intersection in terms of the unknown loads, and following this the principle stresses at each point are calculated.

Several computer programs have been written which do this but they are complex and lengthy.

Because the main displacement is radial and is greatest at the crown cantilever, a considerable simplification can be made by equating the displacement in a radial direction only for a number of arches and a single cantilever.

This program, Known as a “a radial adjustment on the crown cantilever “ is designed to carry out this simplified calculation.

11.2.2	Capabilities And Limitations Of The Program

This type of calculation often makes the assumption of a uniform loading along the arches and it gives a reasonable idea of the behaviour of the structure and a good approximation of the magnitude of the stresses.

It is known, however, that arch loading is almost never uniform, usually the arch loads tend to reduce towards the abutments. Frequently non circular arches are used in practice to take advantage of this to obtain a better stress distribution.

This program has been written to allow for a log spiral shape of arch , which can range from the elliptical, through the circular to the parabolic shape and can closely approximate a three centred arch. The program also allows a reducing arch load from crown to abutment to be specified.

A non circular log spiral shape implies a gradually varying load and the variation of loading specified is rather a matter of experience and judgement. The calculation of arch loading and stresses on the crown is usually remarkably accurate when compared with more rigorous methods for the same structure.

The arches may be of varying thickness from crown to abutment and the radii may vary at every level.

The program will deal equally well with a double curvature dam or a simple constant thickness dam.

It will allow external loads to be applied to the cantilevers to take into account the weight of such structures as ski jump spillways and the hydraulic loads to which they are subjected.

It will allow tailwater loading and uplift pressures to be considered.

11.2.3	Program Data

Starting with the basic constant thickness derived as above, experience has shown that improvements in cantilever stresses can be made by reducing the crest top thickness and sloping the upstream face from the crest to about mid height. This reduces the volume of material in the wall significantly and increases the compressive  stress due to weight on the downstream face, counteracting some of the tension stress in that area. The downstream base of the crown cantilever is thickened  and sloped to meet the vertical face a few metres above plinth level.

At this stage it is necessary to consider the spillway design to ensure that there is adequate width for whatever arrangement is chosen and to also consider the non overflow crest design, bearing in mind that a properly designed upstream parapet can contain high floods and save masonry.

The dam shape is completely defined by nine arch rings and a central cantilever.

The crown cantilever is defined by the distances to the faces for each arc level, with reference to a vertical axis , commonly, but not necessarily, taken as the line from the upstream crown of the crest arch.

The arches are defined in plan by the log spiral shapes selected by the designer, specified by the crown radius of a logarithmic spiral of reference and a Constant ”a”, which defines the rate of change of radius of the spiral with distance from the origin. If “a” is equal to zero the spiral of reference becomes a circle.

The arches need not be symmetrical about the axis of the dam, though they usually are. The crown radius must be the same but the “a” value may be different on either side.

A developed longitudinal section is drawn and from this the distances from crown to abutment are measured. This data, together with the cantilever data is then entered into an Excel spreadsheet, “arcdat.xls.”, which then calculates the radial angles from the central axis of the dam to the abutments and the angle between the foundation slope and the vertical at the abutments at each level. From this information the Constants for calculating the foundation deformation are calculated on the spreadsheet.

The data generated from the spreadsheet is then entered into a data file for the Quickbasic program , “Archcant4”.

The Excel spread sheet and the Quickbasic data files are shown as Figs. 19, 20 and 21.

11.2.4	The Arch Calculations

The mathematics and geometry of the logarithmic spiral are described in Appendix 10.

The arches are analysed in accordance with elastic theory. The two halves of an arch ring of unit depth are divided into voussoir elements, the number of which may be specified by the user, but which should not be less than 25. The deflections at the crown are calculated for unit load at the crown which varies along the length of the half arches according to a specified equation. The deflection is calculated in terms of the unknown crown normal and tangential forces and moments and the applied loading. The deflections of the two halve arches are calculated starting at the abutments with the deformations due to foundation movement and adding the deflections of the successive voussoir elements. They are equated to yield three simultaneous equations, which, when solved, gives the normal force, the shear force and the bending moment at the crown. Each of the arch rings is analysed in turn to give the normal force, the shear force and the bending moments at the crown and abutments and the crown deflection for the unit load at the crown, distributed as specified.

11.2.5	The Crown Cantilever Calculations

The geometric properties of the cantilever are calculated for each level.

The normal forces and bending moments due to the weight of the cantilever are calculated, together with the upstream and downstream stresses at each arch level.

The upstream face of unit width is assumed to have unit triangular loads applied between arch sections and the shear forces and bending moments due to these loads are calculated for each unit load at each level.

The normal forces, shear forces and bending moments due to the total water load are calculated.

Foundation constants are calculated which, when applied to the shear forces and bending moments at the base, allow the radial deflection and angular movement at the base, due to foundation deformation to be calculated.

If it is assumed that the ratios of shear force/area and bending moment/moment of inertia vary linearly between adjacent sections, expressions can be derived for the deflections of a cantilever element bounded by the sections, in terms of the shear force and bending moment at each section, and the distance between the sections. A table of deflection constants is calculated accordingly.

Deflections of the cantilever are calculated from the base upwards.

First the direct radial deflection at the base and the slope at the base due to foundation deformation are calculated.

The total deflection at the next level is made up as follows,

- The slope at the previous level times the difference in level plus

- Deflection of the element under the shears and moments at top and bottom plus,

- Deflections at the previous level.

The total slope at the next level is made up of the following,

- The slope at the previous level plus,

2	The change in slope of the element due to the moment at top and bottom.

and so on proceeding from level to level

## 11.2.6	Solution For The Arch Loads

Formation of the ten simultaneous equations to calculate the unknown arch loads is as follows.

The deflection of the arches at the crown is given by the unknown arch load times the deflection due to the unit arch load, previously calculated.

The cantilever loads are the total loads minus the arch loads.

Therefore the deflection of the cantilever is equal to the deflection due to total loads minus the deflection due to the arch loads.

This may be expressed as an equation at each level giving the deflection  due to each of the unknown arch loads plus the deflection of the arch load at that level which equals the deflection of the cantilever at that level due to the total loads.

The ten simultaneous equations are solved to yield the arch loads at each level.

11.2.7	Loads, Stresses And Deflections

The final cantilever loads and stresses are calculated as follows.

The weight load is added to the other vertical loads to give the total vertical load.

The horizontal cantilever loading is calculated as the total horizontal load minus the arch load.

The cantilever moment is calculated as the moment due to total loads minus the moment due to arch loads plus the moment due to weight.

The cantilever shear force is calculated as the shear force due to total loads minus the shear force due to arch loads.

The sliding factor is calculated as the horizontal shear force divided by the total vertical force.

The upstream and downstream stresses due to these forces and moments are calculated for each level.

The upstream and downstream arch stresses are calculated using the values for thrust, shear and moment previously calculated for unit arch loads and multiplying them by the actual loads.

11.2.8	Estimate Of Cantilever Cracking

Two possible conditions of cracking may be considered:

- Cracking on both faces until no further tensile stresses exist.

- Cracking at he upstream face only until no further tensile stresses exist on that face.

If tensile stresses exist at the base of the upstream face (and they usually do), the section must crack quite substantially to produce no further tensile stress.

To prevent a slow iterative approach to this state, the position of the resultant vertical force is estimated in one bold step and then iteratively corrected.

Assuming the line of thrust produces no tensions the extent of cracking at each level is calculated.

The crown cantilever adjustment is repeated for the cracked cantilever section ,except that its weight is still considered and the arch properties remain unaltered. The cracked concrete is assumed not to exist and to offer no resistance.

Usually 20 or so iterations are required before a cantilever section is found with virtually zero tension stress.

# 11.2.9	General Remarks

The program is written to consider the analysis of nine arches and ten levels, including the base. The base arch characteristics are unknown  but to facilitate the adjustment the deflection of the base arch under unit load is stated as that of the next highest arch times an arbitrary factor.

Nine arches must be included . They may be of variable thickness and asymmetrical.

If the cantilever includes a horizontal step it must be described by two arches a short distance apart.

The adjustment can be carried out under the following conditions.

1	No cracking

2	Cracking on both faces until no tension exists

3	Cracking on the upstream face until no tension on that face exists.

4	Base hinged.

- Cracking on both faces until no tension exists but the upstream cracks

drained to eliminate uplift.

It is possible to specify a different Ec/Er ratio at each abutment of each arch ring and at the base . This allows comparative studies to be made for different foundation conditions.

Stresses calculated by this method tend to be higher than those calculated by more elaborate methods.

The method allows a variety of conditions to be studied and probably sets reasonable extreme limits, within which the true stresses will lie.

The program is fully documented and includes a detailed description of the theory and the calculation method. A copy is held in the Harare office of Stewart Scott Zimbabwe.

11.2.10	The Crown Cantilever Adjustment Results

The initial design of the dam was finalised after numerous crown cantilever analyses of different shapes, to arrive at what was considered to be the least cost solution.

As described above, the geology and topography finally exposed, dictated that the central radius of the spiral be increased from 60 to 80 metres but the previous spiral rate of change of curvature could be retained, as the resulting dam footprint of the dam fitted the sound rock foundation very well.

A new central cantilever profile was selected to take into account a reduction in the width of the  spillway overflow section and a consequent small increase in the design high flood level. Further crown cantilever analyses were carried out to optimise the cantilever profile in detail, to yield minimum quantities and to achieve uniformity of arch loading.

A query was raised by the geotechnical reviewers regarding the effect of a possible variation of the value of Young’s Modulus for the foundation rock,,Er. The selected dam shape was analysed to consider the three cases of Ec/Er with values of 2, 1 and 0.1.

The results of these calculations are fully described in Ref. 10 of the geological reports.

It was found that if the crown cantilever is considered to be cracked, the influence of the value of Ec.Er on the loads and stresses in the structure is not very significant.

The results of the crown cantilever calculations and the tables and graphs illustrating the results are attached to this report as Appendices 11, 12, 13 and 14.

The adjustments have been carried out for the water level at high flood level and for the two conditions ,

- An uncracked cantilever and

- The cantilever cracked at the upstream face only.

Although tension stresses on the downstream face are indicated, they are low and are considered to be within acceptable limits.

The final shape chosen for the dam is completely defined by the logarithmic spiral equation of the spiral of reference and the crown cantilever cross section.

The geometry of the logarithmic spiral is illustrated by the diagram Fig 22 and the Excel spreadsheet, Fig 23. showing the governing equations and calculation table, listing the coordinates of the spiral of reference and the dam faces at distances along the spiral from the origin.

A summary of the loads , forces, stresses and deflections for the two conditions for a value of Ec/Er = 2 is tabulated in the two tables below.

The maximum stress in the masonry at the crown of the arch is 4.72 Mpa for the cracked condition.

The target strength of the masonry mortar is 15 MPa. giving a factor of safety against failure in compression of 3.18

The results for the uncracked and cracked condition are shown in graphical form in Figs 40  to 51.

The chosen shape was then modelled and analysed by the ALGOR finite element program, which confirmed that the stress distribution within the dam was satisfactory and the stresses were within acceptable limits. The deflected shape indicated that the shape in plan and section was well adapted to the site.

It should be noted that in the crown cantilever program compressive stresses are positive and tensile stresses are negative. This is in contrast to the ALGOR finite element program where compressive stresses are negative and tensile stresses are positive.

| CROWN CANTILEVER ANALYSIS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CASE 1 |  |  | Ec/Er = 2 |  | UNCRAC | KED |  |  |  |  |  |
| RESULTS |  |  |  | CANT D/S | CANT U/S | CANT | CANT | CANT | CANT | ARCH U/S | ARCH D/S |
| ARC N0 |  |  | LEVEL | STRESS | STRESS | LOAD | SHEAR | B,M. | DEFLECTION | STRESS | STRESS |
|  |  |  | metres | Mpa | Mpa | Tonnes / m2 | tonnes | ton metres | mm | Mpa | Mpa |
| 1 |  |  | 135 | 0 | 0 | -6.61 | 0 | 0 | -22.62 | 2.79 | 3.8 |
| 2 |  |  | 132.5 | 0.05 | 0.14 | -5.29 | -14.87 | -19.3 | -21.41 | 2.48 | 2.08 |
| 3 |  |  | 128 | -0.33 | 0.63 | 0.33 | -26.04 | -120.8 | -19.2 | 2.29 | 1.78 |
| 4 |  |  | 123 | -0.27 | 0.77 | 4.86 | -13.29 | -254.2 | -16.5 | 2.21 | 1.09 |
| 5 |  |  | 118 | 0.06 | 0.62 | 9.03 | 21.78 | -280.2 | -13.47 | 2.16 | 0.6 |
| 6 |  |  | 114 | 0.46 | 0.43 | 14.80 | 70.65 | -103.2 | -10.85 | 1.83 | 0.38 |
| 7 |  |  | 110 | 1.25 | -0.16 | 19.09 | 140.16 | 312.5 | -8.15 | 1.64 | 0.27 |
| 8 |  |  | 106 | 2.57 | -1.26 | 24.12 | 228.79 | 1043.5 | -5.53 | 1.17 | 0.25 |
| 9 |  |  | 102 | 3.21 | -1.87 | 25.78 | 331.15 | 2146.4 | -3.28 | 0.75 | 0.17 |
| 10 |  |  | 98 | 3 | -1.75 | 30.27 | 446.12 | 3665.5 | -1.56 | 0 | 0 |

| CROWN CANTILEVER ANALYSIS |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CASE 2 |  |  | Ec/Er = 2 |  | CRACKED U/S BASE |  |  |  |  |  |  |  |
| RESULTS |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | CANT D/S | CANT U/S |  | CANT | CANT | CANT | CANT | ARCH U/S | ARCH D/S |
| ARC N0 |  |  | LEVEL | STRESS | STRESS |  | LOAD | SHEAR | B,M. | DEFLECTION | STRESS | STRESS |
|  |  |  | metres | Mpa | Mpa |  | Tonnes / m2 | tonnes | ton metres | mm | Mpa | Mpa |
| 1 |  |  | 135 | 0 | 0 |  | -8.91 | 0 | 0 | -28.05 | 3.45 | 4.72 |
| 2 |  |  | 132.5 | -0.08 | 0.17 |  | -8.06 | -21.21 | 0.27 | -26.91 | 3.12 | 2.61 |
| 3 |  |  | 128 | -0.58 | 0.87 |  | -2.50 | -44.97 | -185.3 | -24.81 | 2.96 | 2.29 |
| 4 |  |  | 123 | -0.75 | 1.24 |  | 1.41 | -48.10 | -451.7 | -22.11 | 2.96 | 1.46 |
| 5 |  |  | 118 | -0.64 | 1.31 |  | 4.64 | -33.13 | -700 | -18.86 | 3.03 | 0.84 |
| 6 |  |  | 114 | -0.68 | 1.53 |  | 10.55 | -1.98 | -778.3 | -15.86 | 2.67 | 0.56 |
| 7 |  |  | 110 | -0.43 | 1.48 |  | 14.36 | 49.11 | -689.3 | -12.48 | 2.51 | 0.41 |
| 8 |  |  | 106 | 0.21 | 1.04 |  | 19.54 | 118.63 | -360.9 | -8.75 | 1.85 | 0.4 |
| 9 |  |  | 102 | 0.86 | 0.43 |  | 21.03 | 210.81 | 236.07 | -4.81 | 1.1 | 0.25 |
| 10 |  |  | 98 | 2.17 | 0 |  | 33.19 | 310.10 | 636.1 | -1.09 | 0 | 0 |

11.3	“Algor” Finite Element Program

11.3.1	The Model

The Geometry of the model is defined by the crown cantilever cross section and the Spiral of Reference of the arch Figs. 21 and 22.

This has to be broken down to a mesh of “brick” elements as defined by the user. In general, for this model, they are 4 metres long along the line of the arch and two metres deep with a width that varies according to the  level of the element but generally as shown on the crown section as shown on Fig 39.

Where the elements abut on to the foundation the element shapes must necessarily be more complex. The foundation elements are extended into the foundation rock by what are termed Vogt Elements which have the same plan area as the foundation elements but the depth of which varies according to the elevation. Every node of the model must be coordinated. The coordination are pre-calculated on two forms of Excel spread sheet. One defines the coordinates in horizontal plan for every level the other defines the coordinates in vertical sections for every level. See Figs. 36 and 37. The model is drawn on the computer by a CAD type programme call “Super Draw”.

The co-ordinates of every node must be entered on the three dimensional drawing with its connections to other nodes defining the “brick’ elements

In the model for Lucilla Poort a horizontal (at the base at the contact between the dam and foundation) crack is modelled which eliminates tensions across the crack and allows water pressure to act on the surface of the crack.

The Vogt Elements are introduced to simulate deformations in the foundation rock beneath the masonry structure down to a depth which is a function of the ratio of Young’s Modulus of masonry to that of rock, Ec/Er. The Vogt Elements are fixed to the rock at the base.

All the model elements are free to rotate and translate in the x, y and z directions except the Vogt Elements at the base which are only free to rotate but with no translation.

In the Lucilla Poort dam model there are 1992 nodes and 1267 brick Elements.

11.3.2	Properties of the Model

The material properties must be defined. Masonry is regarded as concrete and the properties assumed are shown in Table 1 below.

Table 1  :  Material property data input in the LSDDIS

| Gr. | Material | Weight Density N/m3 | Young’s Modulus, E N / mm2 | Poison’s Ratio | Alpha Coeff for Termal Expansion, deg/m | Shear Modulus, G, N/m2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Concrete | 24000 | 2.0E10 | 0.2 | 7.0E.-6 | 6.5E 9 |

11.3.3	Model Loads

The model may be subject to loads due to weight, hydrostatic pressure and temperature changes.

Surfaces are defined by a colour number and a specific number is applicable to those surfaces subject to hydrostatic pressure.

The level of zero hydrostatic pressure is defined by a value called “yref”. This is the level of the water in the dam for the case considered.

“Tref” is defined as the stress free reference temperature. In the cases analysed this is 10ºC.

A temperature drop is given by the difference between Tref and the modal temperatures in the model which must be defined.

A density of water is taken to be 9810 N/m2.

The depth of tailwater is very small and tailwater forces (which would improve the stability are ignored.)

11.3.4	Analysis Procedure

When the model has been defined in “Super Draw” it is then processed by the program to convert it to a Finite Element Model.

Material properties, boundary conditions, loading due to the water load and temperature loading are entered on what is called the “Linear Stress dynamic impact screen”.

The model is then processed to calculate the stresses and deflections, which are displayed in a further model by a programme called “Superview”.

The model may be viewed from any angle and the average stresses at each node are shown as a colour or contour plot.

Other text files are created and the details of forces, stresses at each node may be examined.

The original shape and deflected shape under load can be viewed.

11.3.5	Results

In this report the following load cases are examined.

- A dam with the water level at High Flood Level, RL1238 (theoretical high flood level is RL 1237.95) , in an uncracked state without temperature drop.

- A dam with the water level at High Flood Level, 1238, assumed cracked at the base but without temperature drop.

- A dam with the water level at full supply level, 1235, assumed cracked at the upstream base and with a temperature drop of 10ºC.

- An empty dam, assumed uncracked at the base with a temperature drop of 10ºC.

These represent the extreme load conditions to which the dam is likely to be subjected.  Other conditions were checked using either the Crown Cantilever adjustment or Algor but were not critical to the design and the results are not tabled.  It should be particularly noted that the condition of a dam with a water level at High Flood Level and a temperature drop of 10ºC is not examined as it is not realistic. In Zimbabwe heavy floods in the cool months are unknown.

The shape of the deflected crest in plan is a good indication of the correct choice of curvature of the arch. If it retains the original shape without contraflexure it may be assumed the stresses are well distributed and thrusts are well centred in the structure.

Stress plots of the following type are plotted in Figs 52 to 71.

- A section on the approximate crown of the arch showing Tensor stresses in the y (vertical) direction and showing an exaggerated 	view of the deflected shape.

- A view of the upstream elevation of the dam with tensor stresses on the y, vertical direction.

- A view of the upstream elevation of the dam with tensor stresses in the x, horizontal direction.

- A view of the downstream elevation of the dam with tensor stresses in the y, direction.

- A view of the downstream elevation of the dam with tensor stresses in the x direction.

The table on the following page indicates the maximum compressive and tensile stresses for each view.

It will be noted that the maximum compressive stress is 3.5 MPa.

Tensile stresses are low but if these should give rise to local cracking the load will be transformed to other elements which have ample reserves of strength.

The structure is seen to have a good stress distribution and has a factor of safety of 4.9 against failure in compression.

Table of Algor Stresses and Deflections

| ###### Case | Water Level | Temp. Drop | Tensor Stress Direction | Maximum Compressive stress (Mpa) | Maximum Tensile stress (Mpa) | Crest Deflections (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 (High Flood Level, no temp, uncracked) |  |  |  |  |  |  |
| Cantilever Cross Section | 38 | 0 | Y | 2.50 | 1.80 | 17.53 |
| Downstream Face | 38 | 0 | X | 2.00 | 0.40 |  |
| Upstream Face | 38 | 0 | Y | 0.80 | 1.90 |  |
| Downstream Face | 38 | 0 | Y | 2.50 | 0.43 |  |
| Upstream Face | 38 | 0 | X | 2.50 | 0.10 |  |
| Case 2 (High Flood Level, no temp, cracked) |  |  |  |  |  |  |
| Cantilever Cross Section | 38 | 0 | Y | 2.00 | 0.71 | 20.68 |
| Downstream Face | 38 | 0 | X | 2.00 | 0.10 |  |
| Downstream Face | 38 | 0 | Y | 2.50 | 0.71 |  |
| Upstream Face | 38 | 0 | X | 3.00 | 0.10 |  |
| Upstream Face | 38 | 0 | Y | 1.00 | 0.40 |  |
| Case 3 (Full Supply Level, 10 deg temp, cracked) |  |  |  |  |  |  |
| Cantilever Cross Section | 35 | 10 | Y | 3.50 | 1.00 | 27.95 |
| Upstream Face | 35 | 10 | X | 3.00 | 1.00 |  |
| Downstream Face | 35 | 10 | Y | 1.00 | 0.30 |  |
| Downstream Face | 35 | 10 | X | 2.00 | 1.10 |  |
| Upstream Face | 35 | 10 | Y | 1.00 | 0.29 |  |
| Case 4 (Empty dam, 10 deg temp, uncracked) |  |  |  |  |  |  |
| Cantilever Cross Section | 0 | 10 | Y | 1.50 | 0.43 | 8.84 |
| Downstream Face | 0 | 10 | X | 0.45 | 1.10 |  |
| Downstream Face | 0 | 10 | Y | 0.95 | 0.05 |  |
| Upstream Face | 0 | 10 | X | 0.60 | 1.10 |  |
| Upstream Face | 0 | 10 | Y | 0.30 | 0.43 |  |

12	ABUTMENT STABILITY

  - Arch Thrusts on the Abutments

The arch normal forces, shear forces and bending moments are calculated for each level used in the analysis ‘The calculated magnitude and direction of the resultant thrust of the arches have been tabulated and graphed  in Figs 72 to 76. They are seen to be generally acting normal to the abutment rock contours The arches transmit their thrusts to the abutment rock and it is assumed that the rock mass mobilised will diverge at an angle of 20 degrees on either side of the arch foundations. If the assumption is made that a horizontal joint exists under the abutment and the arch thrust is resisted by the weight of the wall and the abutment mass mobilised, then it is commonly accepted that if the ratio of the resultant thrust to the weight, ( tan phi,) is less than 0.75 the abutments is safe due to the frictional resisting forces alone.

A  calculation of the forces involved and the resulting values of tan Phi as the thrusts extend into the abutment has been made (see Figs  76) they show that the mass mobilised up to 5 m. beyond the ends of the arch crest  would be sufficient to resist the thrust and 25 m. beyond the ends of the crest tan Phi would be reduced to less than 0.3.

Fig  (76) illustrates the variation of tan Phi with distance from the crest ends.

However this concept of a sliding mass is not realistic. No postulated horizontal joint exists and the joint surveys carried out by the Anglo American geotechnical engineers , Mr. D van Rensburg and Dr.A Bye show that there are no joint systems which lead to instability of the abutments, which are entirely in solid ironstone and any wedge systems which may exist would be stable under the arch thrusts which would be resisted by direct compression in the ironstone rock mass. Copies of the reports on the joint surveys are attached as Appendix 15.

  - Stability of the Upper Left Abutment

Although the steep upper left abutment upstream of the arch is in massive ironstone it is jointed.  The rock mass will be consolidated by drilling and grouting in 8 m long by 25 mm diameter anchor bars inclined at 30 degrees to the horizontal and spaced at 1 m vertical intervals.  In plan the bars will intersect the joints and it is visualised that each row will have 5 bars.

The holes will be pressure grouted before inserting the bars and it is intended to produce an impervious, effectively solid block of ironstone as an abutment.  The anchorage will extend from RL 1230 to  RL1238.  (See  Appendix 16)

13	THE SPILLWAY

The site leaves little choice but to design the spillway as a crest overflow structure with a ski jump flip bucket.

The banks are too steep and high to accommodate any other economic form of spillway.

The initial design incorporated a 72 metre wide overflow section but in the light of the abutment conditions exposed it was decided to narrow the width to 56 metres in order to avoid spillage on to the abutment slopes.

The spillway end guide walls have been designed to converge the water by 5 degrees at each side in order to further concentrate the impact zone onto the solid ironstone rock across the flat river bed.

The crest is designed as a Creager profile with a design head of 1.7 metres, which is just under 60 percent of the 3.95 metre design flood rise. This is common practice , for though it leads to sub atmospheric pressures at the high flood discharge the high flows are of short duration and cavitation is unlikely to occur. At the same time it results in an increased value for the coefficient of discharge at all flows.(See Figs 25 and 26).

The Creager profile terminates at a flat flip slab set 2.5 metres below spillway crest level. Experience has shown that the water takes a circular path at this abrupt change of slope with an outside radius of about 3 times the thickness of the falling nappe.

The reduction in design head allows the width of the ski jump structure to be reduced as far as possible, while maintaining a reasonable head to impart forward velocity to the water.

The driving head producing the forward velocity is the difference between the upstream water level and top of the water level on the flat slab. This head varies according to the flow and accordingly the trajectory distance to the river bed varies.

The equations governing the driving head and the form of the trajectory are shown on the Excel spread sheets, Figs. 32 and 33.

The trajectory path is calculated for two values of flow , representing the 1 in 5 year flood

of 50 cumec and the 1 in 2000 year design flood of 651 cumec , in order to bracket the likely extent of the impact zone  range.

The ski jump spillway can be accommodated on the four metre wide arch wall at that level, extended by a further 1.2 metre wide cantilever slab of 400 mm. constant depth. The cantilever shape and width was dictated by considerations of ease of construction and by the Contractor’s availability of existing support frames and formwork.

The forces on the cantilever slab due to the deflection of the water over the slab are calculated from the centrifugal force exerted due to the water travelling in the curved path together with the weight of the water. Typical calculations to determine the velocity and depth of the moving water and pressure exerted on the slab are attached (figs. 34 (a) and (b)). The I in 2000 year outflow flood of 651 cumec. The resultant force on the slab and weir is inclined towards the upstream. It represents a force which reduces stresses in the cantilevers at that level.

The reinforcement is calculated according to the requirements of C.P.114 using the more pessimistic assumption that that the entire potential head of the spillway overflow was transferred to the flip slab as weight and the calculations are shown as Fig. 35.

The spillway is divided into three sections. The two side sections are each 20m long and the central section is 16m long. The crest of the central section is set 100 mm. below the that of the outside sections. This is to concentrate the low normal flows to avoid the unsightly appearance of a dribble flow over the whole length of the spillway. The sections are divided by two substantial splitters which will extend through the nappe. It is proposed to fix targets at the top downstream ends of the splitters to enable deflections of the arch to be measured under flood conditions.

Smaller splitters will be placed at 2.8m centres along the downstream edge of the flip slab in order to aerate the nappe as it leaves the jump.

14	THE PLUNGE POOL

The impact zone of the spillway jet is in massive ironstone but joints do exist and may give rise to uplift pressures under the impact of the water. The area will be cleaned on completion of the dam and if initial remedial measures are necessary to alleviate likely uplift forces they will be taken.

For the following reasons it is not anticipated that a scour hole will develop or if it did that it would be a concern:-

- The maximum specific discharge even in the 10  000 year flood is low - 16 m3/s.m  (typical values would probably be in the range of 60 – 500 m3/s.m on major dams).  A nappe of less than 5-6 metres is regarded as thin whereas on Lucilia Poort the nappe at the cill for the 10 000 year flood is less than 2m.  It is possible that with this low specific disharge, relatively low head and a tailwater in the river downstream that a threshold at which erosion starts is not even reached.

- The cill both aerates the overflow and gives it a horizontal velocity which will further dissipate the energy.

- The floods will all be a very short duration.  During a 10 000 year flood the specific discharge would only be above 10 m3/s.m for 2 hours.  Although the literature suggests that scour holes develop quickly initially, 20 – 50 hours and stop after about 300 hours this happens at specific discharges many times higher than this.

- There are no unusual geological features (dykes or bars) downstream which will cause problems.  In fact the banding of the ironstone and the joints will resist any tendency to cut back towards the wall.

- If there were ever any problems with a scour hole developing it is very easy to get access to the hole developing, unlike situations with major power plants and large river flows, as the dam will not be spilling for most of its life.

However the area will be inspected following floods and remedial works undertaken as required.

Similarly the low water fall, marking the downstream edge of the ironstone dyke, will be inspected and stabilising works will be undertaken if necessary.

If erosion does take place in the plunge pool zone it is unlikely to lead to regressive undercutting of the dam wall.

Experience of spillways of this type, supported by model tests carried out by the Zimbabwe Ministry of Water Development for Odzani and Bangala dams show that the forward velocity of the falling jet leads to a hole progressing downstream from the impact zone.

# 15	THE OUTLET WORKS

15.1	The Bottom Outlet

A 200 mm. diameter stainless steel outlet pipe has been set just above the plinth level, (invert 1200.60). It is located towards the left bank, at a distance of 32.0 metres from the central axis of the dam .It lies under the spillway overhang and when the dam is spilling it will be inaccessible, except at low flows.

The purpose of this pipe is twofold, initially it will be used as a diversion pipe to allow the masonry arch to be constructed without overtopping by the lower flood flows and normal flow.

On the upstream side the pipe is fitted with a reversible galvanised steel screen. If it is wished to close the pipe on the upstream side divers may be sent down to remove the screen which is a sliding fit into a short vertical standpipe. If the screen is reversed it may be used to block the standpipe.

On the downstream side the flow is controlled by a 200 mm. diameter sluice valve. The valve will be protected by a small, concrete valve house structure with a steel lockable door.

On completion of the dam this outlet pipe will be used to pass normal flow to downstream users for primary or environmental purposes.

15.2	The Main Outlet Works

Two 400 mm. diameter stainless steel outlet pipes are set immediately beyond the spillway right hand guide wall. The invert of the pipe is at  RL 1209.50.

On the upstream side the pipes terminate in a short section of plain ended vertical pipe. As for the bottom outlet, the pipes will be fitted with reversible galvanized steel screens which fit into the vertical pipe sections.

On the downstream side the pipes taper down to a 250mm diameter butterfly guard valves which in turn are connected to a 250mm diameter manifold. One branch is fitted with a 250mm diameter sluice valve which will allow controlled flows to be discharged back to the river via an open channel at the toe of the dam. A second branch discharges directly into the suction pipe of the pumpstation which is situated about 330m  down stream from the dam at the end of gorge where it emerges from the hills.

The essential purpose of these pipes is for the abstraction of water to supply the Unki Mine production demand.

A second purpose is to be able to discharge water back to the river if the 200 dia. pipe is out of commision for any reason or to increase the river flow if necessary.

The maximum flow that can be discharged to the river with both the 250 dia. and the 200 dia. fully open is 0.844 cumec with the water level at full supply level,

R.L. 1235.00, reducing to 0.365 cumec with the level at R.L. 1210.00.  The 200 dia. bottom outlet on its own can discharge 0.355 cumec with the water level at full supply level, reducing to 0.174 cumec with the water level at R.L. 1210.00.

If there was no inflow into the dam  it would take approximately 80 days to empty the dam from full, with both pipes fully open.

It is not possible to pass flood water in excess of 0.844 cumec, through the river outlets and control of the water level during the flood season is generally impractical.

In Zimbabwe it is not a requirement to incorporate a large emergency bottom outlet pipe in dams, and indeed to do so is regarded as a potential danger. Mechanical equipment is notoriously subject to lack of maintenance and if a large valve remains closed for a number of years it may not open when required, or having opened, it may not close.

16	ACCESS TO THE DAM

In order to provide access to the completed dam, crest steps are to be constructed up the plinths at the base of the dam. On the left bank a wide and massive plinth has been required in order to stabilize the steep faces of the deep excavations. This ends at level R L 1230  at the base of an abutment block. Wide steps form the upper surface of the plinth and stairs are built into the abutment block leading to the left bank crest walkway.

On the right bank a masonry ramp provides access to the valve house platform at level

R L 1208.3. The right bank excavations are quite shallow and the plinth is narrow. 1m wide steps are built on the plinth up to level R L 1235. Beyond which the slopes are too steep for steps and access to the right bank crest walkway will be by a steel ladder.

- MONITORING OF THE DAM

  - The Dam Behaviour

Measurement of the crest deflection under load is considered to be the most effective way of verifying that the dam behaviour is in accordance with the design predictions.

It is proposed to construct 3 survey beacons with brass base plates which will ensure the fixed centering of whatever survey instrument may be employed to measure the deflections of the targets to be placed on the two spillway splitters.

  - Flow Control

It is necessary to comply with the Water Permit in regard to the passage of normal flow to downstream consumers.

Low flow gauging devices will be required to be installed on the Dwimbika river and the Home Farm Spruit to measure the inflow to the dam. These will comprise notches or flumes. These have not yet been sited but examination of the air photographs indicate that sites do exist, the main problem being the provision of access to the sites.

A gauging weir or flume will be required downstream of the dam to measure the flow passed on to downstream consumers. It had been intended to convert the weir, built by the Contractor for his water supply purposes, to a gauging weir but this has become silted up with flood debris and may no longer be suitable. Alternatives will be studied.

Water delivered to Unki Mine will be recorded by a flow meter installed in the pump station.

17.3	Seepage at adits

Seepage from the right and left bank adits will be measured by suitable devices within the adits and at the portals.

18	GENERAL CONCLUSIONS

The dam structure is well adapted to the topography and geology of the site and its designed shape has resulted in an even stress distribution.

The stresses even in extreme loading conditions are low.

Although a longitudinal upstream foundation crack may develop it will not give rise to any instability, and watertightness of the uncracked section should be assured.

The abutments of the dam are in good solid rock and the arch thrusts are, in general almost normal to the foundation contours.

In accordance with the Terms of the Water Act (Chapter 20 – 24) a Certificate of Safety and Adequacy has been signed by the Approved Engineer, a copy of which is attached herewith. (Appendix 17)

For STEWART SCOTT ZIMBABWE

RMW/mg

*	Attachments